HMSG (Hyundai Motorsport GmbH) — WEC Racing

Administration Guide

Operating the HMSG WEC Racing production environment

| Document | Administration Guide |
| --- | --- |
| Document |  |
| Project | HMSG (Hyundai Motorsport GmbH) — WEC Racing |
| Environment / Region | prd · eu-central-1 (Frankfurt) |
| Stack | hmsg-rac-prd |
| Version | 1.0 |
| Date | 20 August 2026 |
| Owner | HAEE Cloud Team |
| Status | Draft — for review |
| Related | sub Documents in the deliverables folder |
| Baseline | main @ def9a32 |

# 1. Purpose & scope

Day-2 guide for operating the HMSG WEC Racing production environment: the Windows MSSQL host, AWS Client VPN access, and (when enabled) the Site-to-Site VPN.

# 2. Access paths

| Path | How to use | Notes |
| --- | --- | --- |
| AWS Client VPN | Import HSMG-client-config.ovpn into the AWS Client VPN client and sign in with Entra ID SSO — see the Client VPN Quick-Start for end users | Primary access for developers (≤10 users) |
| RDP to EC2 | mstsc → MssqlPrivateIp (172.200.0.128–159 range) with sysadm | Only reachable through Client VPN / on-prem — no public IP |
| SSM Session Manager | EC2 console or aws ssm start-session; uses the instance profile | Backup access when RDP is unavailable |
| AWS console / CLI | AWS account 440027026402, eu-central-1, stack hmsg-rac-prd | Operations and troubleshooting |

# 3. Key resources

| Resource | What it is |
| --- | --- |
| hmsg-rac-prd-vpc-ec1 | VPC 172.200.0.0/24 — dual-AZ public / private / DB subnets |
| hmsg-rac-prd-mssql01-ec2-ec1a | Windows Server 2022 (m7i.2xlarge) — 250 GB root + 4 TB D: data volume |
| hmsg-rac-prd-clientvpn-ec1 | Client VPN endpoint — SAML (Entra ID) auth, UDP 443, split tunnel |
| hmsg-rac-prd-vpn-conn-ec1 | Site-to-Site VPN (optional) — static route to the on-prem network |
| hmsg-rac-prd-sysadm-password | Secrets Manager secret — sysadm local administrator password |

# 4. Operating rules

UserData runs only at first boot: rotate sysadm by changing the Windows password on the host first, then update the secret.

The DB subnet has no internet route by design — AWS APIs are only reachable through the VPC interface endpoints.

Do not attach a public IP to the EC2 instance.

Entra ID application and user management are client-owned after handover.

Temporary implementation IAM permissions are removed at project completion.

Client VPN → CloudWatch (90 days).

# 5. Routine checks

| Check | Expected |
| --- | --- |
| Client VPN endpoint state | available; clients connect via Entra ID SSO |
| 4 TB data volume | Mounted as D:, NTFS, label SQLData, at least 20% free for SQL growth |
| Port 1433 reachability | Allowed from 10.200.0.0/22 (and on-prem CIDR when the S2S VPN exists) |
| Site-to-Site tunnel state | UP on both ends; route advertised in pri/db route tables |
| Secrets Manager secret | Matches the current Windows password |

# 6. Handover

At project completion: deliver the documentation set, remove temporary IAM access, transfer VPN user administration to HMGRS / the client, and obtain sign-off.

Prepare for handover: export Security Group rules, route tables and endpoint IDs from the resource inventory workbook before transferring ownership.